QUEUE Design

Design Documentation

Version 1.0

July 2024

Table of Contents

Design Overview	2

Functional Description	3

Size Control	3

Peak operation	3

The value on req_out is assigned to the FIRST IN request in the queue which means the oldest request.	3

Push operation	3

Pull operation	3

Outputs Signals	3

Signal description	4

Queue block	4

List of Figures

Figure 1 : block diagram of the queue design	2

List of Tables

Table 1  Queue block signals	4

Design Overview

The queue block is a queue with maximum physical size of 2QUEUE_SIZE, where QUEUE_SIZE is a parameterized value, and a programmable maximum value called almost full.

Figure 1: block diagram of the queue design

Functional Description

Size Control

- The size can be controlled using almost_full value. Where the queue limits pushing values according to the almost full. The value of almost_full is inverted using 2’s compliment meaning that if almost_full = zero then the size will be  2QUEUE_SIZE which is the maximum size, and if almost_full =  2QUEUE_SIZE−1  then the size will be 1 which is minimum size.

Peak operation

The value on req_out is assigned to the FIRST IN request in the queue which means the oldest request.

Push operation

- Push operation is done by asserting the push signal and writing the value to the req input then when the push_ready is asserted the value is pushed to the queue.

Pull operation

- Pull operation is done by asserting the pull signal then when the pull_ready is asserted the FIRST IN is pulled from the queue.

Outputs Signals

- The outputs signal represent the current state of the queue block. full is 1 if the size of the queue reached or exceeded the limit determined by almost_full. empty is 1 if there is no requests in the queue. size represents the current number of requests in the queue. size_exceeded_intr is asserted when there is push while the maximum size of the hardware is reached, the size_exceeded_intr stay asserted until resetting. push_ready allows push if and only if the size is less than the limit determined by almost_full. pull_ready allows pull if and only if there is at least 1 request in the queue.

Signal description

The main signals:

clock - oscillating between high and low state and synchronizing actions in the circuit.

reset - which sets all the values to a predetermined state.

Queue block

| Port Name | Size | I/O | Description |
| --- | --- | --- | --- |
| req_in | 1 request | I | Request input. |
| req_out | 1 request | O | Request output from the queue. |
| pull | 1 | I | Pull signal used in pull operation. |
| pull_ready | 1 | O | Pull ready signal used in pull operation. |
| push | 1 | I | Push signal used in push operation. |
| push_ready | 1 | O | Push ready signal used in push operation. |
| full | 1 | O | Full status output. |
| empty | 1 | O | empty status output. |
| occupancy | QUEUE_SIZE+1 | O | occupancy status output. |
| almost_full | QUEUE_SIZE | I | Almost full input. |
| size_exceeded_intr | 1 | O | Size exceeded interrupt. |

Table 1 Queue block signals